# DESIGN AND SIMULATION OF AN 8-BIT ALU

Objective:

To design and verify a 8-bit Arithmetic Logic Unit (ALU) implementing basic arithmetic and logical operations using Verilog HDL.

Software used:

Visual Studio Code, Icarus Verilog and GTKWave.

Methodology:

| Combinational | Sequential |
| --- | --- |
| Output only depends on the present input. | Output depends on the present input and past output. |
| Memory element is absent. | Memory element is present. |
| No clock signal is applied. | Clock signal is required. |
| Combinational | Memory Combinational |
| Example: Half adder, Full adder, Multiplexer | Example: Flipflops, Counters, Registers |

The ALU was modeled using combinational logic in Verilog. A testbench was developed to check the functionality of the main code. Functional verification was performed using iVerilog and GTKWave.

Operations:

| Opcode | Operation |
| --- | --- |
| 000 | A + B (Addition) |
| 001 | A - B (Subtraction) |
| 010 | A & B (AND) |
| 011 | A ¦ B (OR) |
| 100 | A^B (XOR) |
| 101 | ~A (NOT) |
| 110 | A<<1 (Logical left Shift) |
| 111 | A>>1(Logical Right Shift) |

Verilog Code:

module alu (

input [7:0] A,

input [7:0] B,

input [2:0] opcode,

output reg [7:0] result,

output reg carry,

output reg zero

);

always @(*) begin

carry = 0;

case (opcode)

3'b000: {carry, result} = A + B;        // ADD

3'b001: {carry, result} = A - B;        // SUB

3'b010: result = A & B;                 // AND

3'b011: result = A | B;                 // OR

3'b100: result = A ^ B;                 // XOR

3'b101: result = ~A;                    // NOT

3'b110: result = A << 1;                // LSHIFT

3'b111: result = A >> 1;                // RSHIFT

default: result = 8'b0;

endcase

zero = (result == 0);

end

endmodule

Test bench:

module alu_tb;

reg [7:0] A, B;

reg [2:0] opcode;

wire [7:0] result;

wire carry, zero;

alu uut (

.A(A),

.B(B),

.opcode(opcode),

.result(result),

.carry(carry),

.zero(zero)

);

initial begin

$dumpfile("alu.vcd");

$dumpvars(0, alu_tb);

A = 8'd10; B = 8'd5;

opcode = 3'b000; #10;  // ADD

opcode = 3'b001; #10;  // SUB

opcode = 3'b010; #10;  // AND

opcode = 3'b011; #10;  // OR

opcode = 3'b100; #10;  // XOR

opcode = 3'b101; #10;  // NOT

opcode = 3'b110; #10;  // LSHIFT

opcode = 3'b111; #10;  // RSHIFT

$finish;

end

endmodule

Expected outcome:

| Opcode | Operation | A | B | Expected Result (R) |
| --- | --- | --- | --- | --- |
| 000 | A + B | 10 | 5 | 15 |
| 001 | A - B | 10 | 5 | 5 |
| 010 | A & B | 10 | 5 | 0 |
| 011 | A ¦ B | 10 | 5 | 15 |
| 100 | A^B | 10 | 5 | 15 |
| 101 | ~A | 10 | - | 5 |
| 110 | A<<1 | 10 | - | 20 |
| 111 | A>>1 | 10 | - | 5 |

Calculations:

In decimal, A = 8’d10; B = 8’d5

In binary, A = 4’b1010; B = 4’b0101

In hexadecimal, A = 0xA; B= 0x5

Addition:

In decimal, A + B = 10 + 5 = 8’d15

In hexadecimal, A + B = 0xA + 0x5 = 0xF

Subtraction:

In decimal, A - B = 10 + 5 = 8’d5

In hexadecimal, A - B = 0xA + 0x5 = 0x5

AND operation:

In binary,

A = 1010

B = 0101

R = 0000

In hexadecimal, R = 0x0

OR operation:

In binary,

A = 1010

B = 0101

R = 1111

In hexadecimal, R = 0xF

XOR operation:

In binary,

A = 1010

B = 0101

R = 1111

In hexadecimal, R = 0xF

NOT operation:

In binary,

A = 1010

~A = R = 0101

In hexadecimal, R = 0x5

Left Shift by 1-bit:

In binary,

A = 0000 1010

R = 0001 0100

In hexadecimal, R = 0x14

Right Shift by 1-bit:

In binary,

A = 0000 1010

R = 0000 0101

In hexadecimal, R = 0x5

Results: